## Operating limits: ITT

Normal idle range			400-660°C

Normal operating range		400-695°C

Max @ shutdown w/cond.lever	610°C

Max continuous			695°C

Max acceleration			825°C

Hot start				925°C (2 sec)

Max during starts			1090°C

Measures ITT                8 Thermocouples

Prop RPM (N2):

Idle operating range		<1800RPM

Normal operating range		2200 RPM ±25

Over speed governor		2332 RPM ±40

Fuel topping governor		2398 RPM

Engine Tachometer (N1):

Introduce fuel			@ 12%

Flameout				10-12%

FCU stuck @min.flow(rollback)	40-60%

Minimum for flight (griped)	60%

Idle operating range		62-65%

Normal operating range		89-93%

Maximum				101.5%

Max acceleration			102.6%

## Airspeed Limitations:

Max clean (surface-20,000’)	280 KIAS

(20,000- 25,000’)	245 KIAS

Max canopy emergency open	250 KIAS

Max canopy open			240 KIAS

Max turbulent air penetration	195 KIAS

Max gear extension		150 KIAS

Max flap extension		120 KIAS

Max gear retraction		120 KIAS

Min flap retraction		90 KIAS

Downwind airspeed			100 KIAS

VFR/overhead break airspeed	170 KIAS

Full flap landing airspeed	85 KIAS

No flap landing airspeed	95 KIAS

Best angle of climb (Vx)	75 KIAS

Best rate to climb (Vy)		100 KIAS

Best Maneuvering Speed (Va)	135 KIAS

Recommended bailout airspeed	90-120 KIAS

Max range airspeed		580 ft-lbs

Max endurance airspeed		420 ft-lbs

## Torque:

Idle operating range		150 ft-lbs

Auto ignition range		300-180…

Descent				300…

Prop over speed test range	400-500…

Normal operating range		400-l015…

Climb					1015 (695°C)

Max allowable			1015…

Maximum torque			1315…

Max torque (oil malfunction)	850…

## Fuel Flow:

Idle					~125 lbs/hr

Normal operation			~335 lbs/hr

Fuel Type				JP5

Fuel Alternatives			JP4/ JP 8

Oil Temperature:

Min starting temperature	-40°C

Normal Operating range		10-99°C

Maximum				100°C

## Oil pressure:

Idle operating range		40 PSI min

Normal operating range		65-85 psi

Maximum fluctuation		+5 (w/limits)

Max oil pressure			100 psi

Dynamic Maneuvers			10 psi for 5s

Cold Days				80 to 100 psi

## G Limits:

Up to 280 KIAS			+4.5/-1.0 G's

Up to 220 KIAS			+4.5/-2.3 G's

Flaps down				+2.0/-1.0

Gear down                     +1.5/-0.0

## Time limits:

Max time inverted			15 sec

Max time vertical (nose up)	15 sec

Max time vertical (nose down)	3 sec

Max time wings vertical (knife edge):3 sec

“Zero G” condition		transient

Starter		40s/60s/40s/60s/40s/30min

Dimensions:

Wing span				33’ 5”

Length				28’8”

Height (at rest)			9’11”

Prop arc clearance	11”,(7.6” diameter)

## Weight limitations:

Maximum ramp weight 		4,425 lbs

Takeoff and landing 		4,400 lbs

Max aerobatic weight 		4,300 lbs

Max zero fuel weight 		3,650 lbs

Approximate basic weight	~2,970 lbs

## Miscellaneous limits:

Max sink rate @ touch down	600 FPM

Max Total Winds	25

Max crosswind component	22(NF),15(FF)

Max operating altitude		25,000’

Air start envelope, SL-20k’	80-280 KIAS

Engine: Pratt & Whitney PT6A-25

Flat rated				550 SHP(1315)

Normal operation			425 SHP(1015)

Stages		   3 axial, 1 centrifugal

Compression Ratio			7:1

Reduction ratio               15:1

Compr.Turbine (Gas)       37,000rpm (N1/NG)

Power Turbine (Free)      33,000rpm (N2/NF)

## Landing Gear:

Nose gear turn limits		±30°

Main tire pressure		90 psi ± 2

Nose wheel tire pressure	70 psi ± 2

Main Oleo Height			3 inches

Nose Gear Oleo Height		3-5 inches

Red Tire Mark max slip		¼ inch

Aldis Light    Ground       In Flight          Blade Angles:

Steady Green   Takeoff      Land               Feathered                     87.5

Flash Green    Taxi         Return to land     Low Pitch/ Idle               17.5

Steady Red     Stop         Give way, circle   Beta                          -5

Flash red      Taxi clear   Do not land

Flash White    Return to Starting Point        Torque Failure:    N1%        Fuel Flow

Red + Green    Use Extreme Caution             205 ftlbs          77-80      150-170

Pyrotechnic    Waveoff                         650 ftlbs          85-87      200-220

850 ftlbs          90         250-270

# Oil Pumps/ Misc. Oil:

Internal scavenge (Pulls from Accessory Gear Box/ Compressor)

External scavenge (Pulls from Reduction Gear Box/ Power Turbine)

Main Oil Pump

Prop Governor Gear Type Pump                   Glide Characteristics:

Oil System Capacity         4.4 gal            Clean   FTHR     100K   800fpm    12:1

Oil Tank Capacity           2.8 gal            G      FTHR     87K    1200fpm    8:1

Usable Oil (upright)        1.4 gal            F      FTHR     93K    1250fpm    8:1

(inverted)       .25                Dirty   FTHR     88K    1650fpm    6:1

Clean   UNFTHR   117K   2400fpm    3:1

# Generator/ Battery/ GPU:

GPU output needed for start 28v 600 amps       Survival Gear:

Battery rated @             24v 24 amps        Radio         PRC 90,   282.8 SAR, T/R

Generator rated @           30 vdc 200 amps                            243   Guard, T/R

Starter rated @             28vdc 800 amps                             243   Beacon

Gen. Control Panel          27-29.5 v                                  Range= 60 miles

Overvoltage protection @    34 v               Strobe Light  SDU5-E

Undervoltage Protection @   25 v               Smoke Flare   Mark 124, Day- smoke, 1 Ring

Annunciator light on @      25-27.5 v                                  Night-flare,2 Ring

Battery for Start           over 22 v          Pencil Flare  Mark 70,  Range= 200ft

Min Battery During Start    over 10v

Replace Battery below       18v                Landing Gear Switches:

GPU Start                   b/w 18-22v         Left  Saftey  Gear Warning, Nacws Test

Ampmeter w/ GPU             Discharing               Up      Position, Condensor

Down    Position, Peanut light

# Prohibited Manuvers (anytime):                 Right Saftey  Disables Gear Up, AOA

Night Form Flights (only during emergency)           Up      Position

Inverted Stalls                                      Down    Position, Peanut light

Inverted Flight above 220 KIAS                 Nose  Saftey  Beta, Disable Rudder Shakers

Intentional Spins with G/F Down                      Up      Position, Land Light Anunc.

Intentional Inverted Spins                           Down    Position, Peanut Light

Intentional Spins Feathered

# Miscellaneous:

Prohibited Maneuvers (solo):                   Flap extension   30 up/down,10s to extend

Emergency Procedures, PPELS                    4 EPL Don’ts     Starts,Jam Accelerations,

Spins                                                           Beta,Advancing past point

Stalls                                                          at which fluctuations are

*Only previously introduced maneuvers                           min during overspeed cond

Left Air Intake  Acc. Gear Box, Oil Cooler

Fuel System:                                                    Cockpit Air, Starter Gen.

Fuel Nozzles                  10 Pri./4 Sec.   Oil/Fuel Heater     Regulates b/w 70-90 F

FCU PSI                       800 PSI          Oil Cooler bypass   Opens @60, Closes @71

PCL spool time                5 s              4 Gyros?         2 Turn, 1 Dir.,1 Attitude

Primary Pump operating time   10hrs Solitary   4 Prop Gov Parts Flyweights,Speeder Spring

Fuel Drains:                  5 + 1 Firewall                    Pilot Valve, Oil TypePump

Fuel Quantity:          6 Capacitance Probes   Pitch Change     Feather Spring, Counter-

Activates Low Fuel Light:     2 Thermistors                     Weights,Servo Piston,Beta

Fuel Capacity                 135 gal                           Ring, High Press Oil

Usable Fuel                   130 gal          Open Canopy      Increases Rate of Descent

By 300 f/pm

Flux Detector    Located in Left Wing

# Oxygen System:					     Air Conditioning Switches:

Oxygen system capacity		1850 Psi@70°F    High Pres.(black)   380 psi from condenser

Oxygen system volume          76 cubic feet    Low Pres. (copper)    3 psi to compressor

O2 relief valve ruptures @    2775 psi

# Engine Inlet Bypass Door:                      Aircraft Discrepancy Book:

Can’t close above             150 KIAS         Min Discrepancy records  last 10 flight

Opens automatically @         200 KIAS         Turnaround on A sheet    24hrs,1 flight

* Only Power Plant Ice Protection System       Daily on A sheet         72hrs,or 24hrs

since last flight

Icing Checklist:                               Six Checks               7 Day Parachute +

Pitot Heat On                                                           First Aid, 7 Day

Cockpit Enviro & Defog as Req.                                          Lube, 14 Day Wash

Engine Bypass Door Open                                                 56 Day Corrosion,

Auto Ignition Switch On                                                 25 hr Oil Sample

# Weather Minimums for Solo:                     Spin Characteristics:

Winds                         25 KIAS Steady   Erect Spin             80-100 KIAS Steady

10 KIAS Xwind                           AOA Pegged 30 units

No Tail Wind                            Turn Needle Pegged

Homefield Clouds              1500 ft/ 3 vis   Inverted Spin          0 KIAS

Working Area Clouds/ PA’s     3000 ft/ 3 vis                          AOA 3 units

8000 ft/ 3 vis                          Turn Needle Pegged

Landing Gear Warning Indications:              Rate of Descent        9000-12000 f/pm

GF                        Wheel Warn. Light  Rate of “ per turn     375-500 f/pm

Warning Horn

G, PCL Below 75% N1        Gear Handle Light  Misc Requirements:

Wheel Warn. Light  Takeoff Min Fuel       300 lbs per side

Warning Horn       Landing Min Fuel       150 lbs per side

Gear Handle Up on Ground    Gear handle Light  Temp/ Dew Point        4 Max Spread

Wheel Warn. Light  Runway Lengths         3000 Dry VMC, (75w)

Warning Horn                              4000 Wet VMC,(100w)

Handle & Gear Disagree      Gear Handle Light                         4000 Touch n Go

# Engine Failure Indications:

Condition      Torque    N1       ITT      RPM      FF      Oil Temp. Oil Pres.

Flameout       0         10-12    200      200      50      Decr.     Various  10 Psi

Rollback       Low       40-60            <2200     80-100  Norm.     Norm.

Compr. Stall   Fluct.    Fluct.   Fluct.  Surges    Fluct.  Norm.     Norm.

# NACWS:

Test                    +/- 5 heading, +/- 125 ft of altimeter

Tracking                50 Transponder Aircraft up to 20NM

Passive Mode            Monitors Radar and Transponders up to 20NM

Active Mode             Monitors own Interrogations up to 6NM, works when Passive is

Not working, up to 8 Traffic Advisories

Enroute Mode            Proximity Zone = 20NM, +/- 2700’

Protection Zone = 1NM, +/- 500’, A/C w/n 20s (6 tones in 2s)

Landing Mode            Proximity Zone = 1.5NM, +/- 500ft

Protection Zone = .1NM, +/- 500ft, A/C w/n 10s (12 tones in 2s)

DME Ranges              20, 10, 5, 3, 1.5 (DME is prim. Display shown when no traffic)

Range Button            Toggles b/w Enroute and Landing Modes

Proximity Screen (45s)  = 900 to 2700ft below            = same altitude, +/- 300 ft

= 900 to 2700ft above            = unknown altitude

Systems Test:  TACAN 180°+/-3.5°               GPS:  34.5 NM

VOR 000° – 005°                       CDI ½ right

DME 0 – 0.1                           OBS OUT 315°

RMI 130°

Altimeter +/- 75 ft of field

Prop RPM 1950-2150